Faculty of Engineering and Technology

Department of Electrical and Computer Engineering

ENCS5399 - Control Systems

Assignment

Prepared By:

Aya Dahbour                          1201738

Instructor: Dr. Mahran Quraan

Question 1:

Code:

clc;

clear;

close all;

% Define transfer function G(s) = 2 / (s*(s+3))

num = 2;

den = conv([1 0], [1 3]);  % s*(s+3) = s^2 + 3s

% Create transfer function

G = tf(num, den);

% Create unity feedback system

sys = feedback(G, 1);

% Plot step response

figure;

step(sys);

title('Step Response of Unity Feedback System');

xlabel('Time (sec)');

ylabel('Amplitude');

grid on;

% Display poles for verification

disp('Poles of the closed-loop system:');

disp(pole(sys));

Question 2:

Code:

clc;

clear;

close all;

%% Define the system

num = 9;

den = conv([1 0], [1 2]); % s(s+2) = s^2 + 2s

G = tf(num, den);

% Closed-loop system with unity feedback

sys = feedback(G, 1);

%% Plot Step Response

figure;

step(sys);

title('Step Response');

xlabel('Time (sec)');

ylabel('Amplitude');

grid on;

%% Step Response Characteristics

info = stepinfo(sys);

fprintf('Rise Time (Tr): %.4f sec\n', info.RiseTime);

fprintf('Settling Time (Ts): %.4f sec\n', info.SettlingTime);

fprintf('Peak Time (Tp): %.4f sec\n', info.PeakTime);

fprintf('Overshoot (OS): %.2f%%\n', info.Overshoot);

%% Poles, damping ratio and natural frequency

poles = pole(sys);

disp('Poles:');

disp(poles);

wn = abs(poles(1));  % natural frequency

zeta = -real(poles(1))/wn;  % damping ratio

fprintf('Natural Frequency (wn): %.4f rad/s\n', wn);

fprintf('Damping Ratio (zeta): %.4f\n', zeta);

Question 3:

clc;

clear;

% Given uncompensated settling time

Ts_uncomp = 0.9434;

% Required compensated settling time

Ts_comp = 1.05 * Ts_uncomp;

fprintf('Target compensated settling time: %.4f sec\n', Ts_comp);

% PI controller design (trial values)

Kp = 0.01;

Ki = 0.1;

fprintf('Designed PI controller gains: Kp = %.4f, Ki = %.4f\n', Kp, Ki);

Question 4:

Question 5:

clc;

clear;

close all;

%% Task 1: Root Locus Plot

disp('--- Root Locus ---');

num = 1;

den = conv(conv([1 2],[1 10]),[1 15]);

G = tf(num, den);

figure;

rlocus(G);

title('Root Locus of G(s)');

grid on;

%% Task 2: Desired Pole and Gain Calculation

disp('--- Desired Pole & Gain ---');

OS_percent = 7;

OS = OS_percent/100;

Ts = 0.9434;

zeta = -log(OS) / sqrt(pi^2 + (log(OS))^2);

wn = 4 / (zeta * Ts);

sigma = zeta * wn;

wd = wn * sqrt(1 - zeta^2);

desired_pole = -sigma + 1j*wd;

fprintf('Damping ratio: %.4f\n', zeta);

fprintf('Natural frequency: %.4f rad/s\n', wn);

fprintf('Desired dominant pole: %.4f %+ .4fj\n', real(desired_pole), imag(desired_pole));

G_at_s = evalfr(G, desired_pole);

K = abs(-1 / G_at_s);

fprintf('Calculated gain K: %.4f\n', K);

%% Task 3: PI Controller Design

disp('--- PI Controller Design ---');

Ts_uncomp = 0.9434;

Ts_comp = 1.05 * Ts_uncomp;

fprintf('Target compensated settling time: %.4f sec\n', Ts_comp);

Kp = 0.01;

Ki = 0.1;

fprintf('Designed PI controller gains: Kp = %.4f, Ki = %.4f\n', Kp, Ki);

%% Task 4: Step Response for both systems

disp('--- Step Responses ---');

G_open = K * G;

sys_uncomp = feedback(G_open, 1);

figure;

step(sys_uncomp);

title('Uncompensated System Step Response');

grid on;

Gc = tf([Kp Ki], [1 0]);

G_comp_open = series(Gc, G_open);

sys_comp = feedback(G_comp_open, 1);

figure;

step(sys_comp);

title('Compensated System Step Response');

grid on;

figure;

step(sys_uncomp, sys_comp);

title('Comparison: Uncompensated vs Compensated');

legend('Uncompensated','Compensated');

grid on;

1.

2.

3.

4.

5.

-  Compensated System:

-  Uncompensated System: