Faculty of Engineering and Technology

Department of Electrical and Computer Engineering

Computer Architecture - ENCS4370

Project #2 report

First Semester – 2024/2025

Design and Verification of a Simple Multi-Cycle RISC Processor in Verilog

Prepared by:

Rama Abu Hnaish – 1211751

Aya Dahbour – 1201738

Yara Obaid – 1212482

Instructor: Prof. Ayman Hroub

Date: 14/1/2025

Section: 1 & 3

# Abstract

The project focuses on creating and testing a simple 16-bit RISC processor using Verilog. The processor is designed with a custom Instruction Set Architecture (ISA) that supports R-Type, I-Type, and J-Type instructions. It includes eight general-purpose registers, a program counter (PC), and a return register (RR) to manage function calls. The design also features separate memory units for instructions and data. Using a many stages including fetch, decode, ALU, memory access, and write-back—the processor achieves efficient execution of instructions.

This report explains how the datapath and control signals were built and how the different parts of the processor work together. It also shows the results of the design and includes the Verilog code.

# 1. Design Specifications

## Motivation

This project is inspired by the principles of the RISC (Reduced Instruction Set Computer) architecture and focuses on the design of a Multi-Cycle Processor. The multi-cycle approach allows each instruction to be executed over multiple clock cycles, enabling efficient resource utilization while simplifying control logic. This design is well-suited for educational purposes, balancing complexity with clarity in understanding processor operation.

Key features of this multi-cycle processor include:

- Each instruction is 16 bits in length, optimizing instruction memory usage.

- The processor features eight 16-bit general-purpose registers for performing integer operations.

- A 16-bit Program Counter (PC) tracks the flow of program execution.

- A dedicated 16-bit Return Register (RR) is used for function call management, storing return addresses.

- The instruction set supports three types of instructions: R-type, I-type, and J-type, providing flexibility and functionality.

- Two separate physical memory units are implemented, one for instructions and one for data, improving memory access efficiency.

- The Arithmetic Logic Unit (ALU) performs a variety of operations and generates status signals to influence control flow.

- The multi-cycle design reduces hardware duplication by reusing functional units across cycles, improving cost-effectiveness and simplicity.

## Instruction Formats

The instruction set has the following instruction formats:

### 1.2.1 R-Type (Register Type)

| Opcode (4 bits) | Rd (3 bits) | Rs (3 bits) | Rt (3 bits) | Function (3 bits) |
| --- | --- | --- | --- | --- |

- 4-bit opcode: opcode.

- The opcode is zero for all R-Type instructions.

- 3-bit Rd: destination register

- 3-bit Rs: first source register

- 3-bit Rt: second source register

- 3-bit: Function. This field determines the specific operation of the instruction.

### 1.2.2 I-Type (Immediate Type)

| Opcode (4 bits) | Rs (3 bits) | Rt (3 bits) | Signed Imm (6 bits) |
| --- | --- | --- | --- |

- 4-bit opcode: opcode.

- 3-bit Rs: first source register

- 3-bit Rt: destination register

- The immediate value is zero-extended for logical instructions and sign-extended for all other

- instructions.

- In the case of BEQ and BNE instructions, which are I-type instructions, the branch target is calculated as

- follows:

- Branch target = Current PC + sign extended immediate

### 1.2.3 J-Type (Jump Type)

| Opcode (4 bits) | 9-bit offset | Function (3 bits) |
| --- | --- | --- |

- 4-bit opcode: opcode.

- The opcode is 1 for all J-Type instructions.

- 3-bit: Function. This field determines the specific operation of the instruction.

- The jump target address is calculated by concatenating these two fields: PC [15:9], 9-bit offset.

## 1.3 Instruction Set

The table below details the various instructions that have been incorporated into the CPU architecture.

| Instruction | Meaning | Opcoe Value | Function Value |
| --- | --- | --- | --- |
|  | R-Type Instructions |  |  |
| AND Rd, Rs, Rt | Reg(Rd) = Reg(Rs) & Reg(Rt) | 0000 | 000 |
| ADD Rd, Rs, Rt | Reg(Rd) = Reg(Rs) + Reg(Rt) | 0000 | 001 |
| SUB Rd, Rs, Rt | Reg(Rd) = Reg(Rs) - Reg(Rt) | 0000 | 010 |
| SLL Rd, Rs, Rt | Reg(Rd) = Reg(Rs) << Reg(Rt) | 0000 | 011 |
| SRL Rd, Rs, Rt | Reg(Rd) = Reg(Rs) >> Reg(Rt) | 0000 | 100 |
|  | I-Type Instructions |  |  |
| ANDI Rt, Rs, Imm | Reg(Rt) = Reg(Rs) & Imm | 0010 | NA |
| ADDI Rt, Rs, Imm | Reg(Rt) = Reg(Rs) + Imm | 0011 | NA |
| LW Rt, Imm(Rs) | Reg(Rt) = Mem(Reg(Rs) + Imm) | 0100 | NA |
| SW Rt, Imm(Rs) | Mem(Reg(Rs) + Imm) = Reg(Rt) | 0101 | NA |
| BEQ Rs, Rt, Imm | if (Reg(Rs) == Reg(Rt)) Next PC = branch target else Next PC = PC + 1 | 0110 | NA |
| BNE Rs, Rt, Imm | if (Reg(Rs) != Reg(Rt)) Next PC = branch target else Next PC = PC + 1 | 0111 | NA |
| FOR Rs, Rt | • Rs stores the loop target address, i.e., the address of the first instruction in the loop block • Rt stores the initial number of the loop iterations, i.e., the initial value of the loop counter • The Rt register is decremented at the end of each iteration. The loop exits when the content of the Rt register becomes zero • The immediate field is ignored in this instruction | 1000 | NA |
|  | J-Type Instructions |  |  |
| JMP Offset | Next PC = jump target | 0001 | 000 |
| CALL Offset | Next PC = jump target PC + 1 is stored on the RR | 0001 | 001 |
| RET | Next PC = value of the RR The 9-bit field is ignored in this instruction | 0001 | 010 |

# 2. Design and Implementation

### 2.1 Register-Transfer Level (RTL)

### R-Type Instructions

- AND: Reg(Rd) = Reg(Rs) & Reg(Rt), Opcode: 0000

- ADD: Reg(Rd) = Reg(Rs) + Reg(Rt), Opcode: 0001

- SUB: Reg(Rd) = Reg(Rs) - Reg(Rt), Opcode: 0010

- SLL: Reg(Rd) = Reg(Rs) << Reg(Rt), Opcode: 0011

- SRL: Reg(Rd) = Reg(Rs) >> Reg(Rt), Opcode: 0100

- IF: Instruction ← MEM[PC], PC = PC + 1

- ID: data1 ← Reg(Rs), data2 ← Reg(Rt)

- EX: ALU_result ← OPCODE(data1, data2)

- WB: Reg(Rd) ← ALU_result

### I-Type Instructions (Immediate Operations)

- ADDI: Reg(Rt) = Reg(Rs) + Imm, Opcode: 0011

- ANDI: Reg(Rt) = Reg(Rs) & Imm, Opcode: 0010

- IF: Instruction ← MEM[PC], PC = PC + 1

- ID: data1 ← Reg(Rs), Imm ← Sign_Extender[Instruction[5:0]]

  - For ANDI: Imm ← Zero_Extender[Instruction[5:0]]

- EX: ALU_result ← OPCODE(data1, Imm)

- WB: Reg(Rt) ← ALU_result

### I-Type Instructions (Memory Access)

- LW: Reg(Rt) = MEM[Reg(Rs) + Imm], Opcode: 0100

- SW: MEM[Reg(Rs) + Imm] = Reg(Rt), Opcode: 0101

- IF: Instruction ← MEM[PC], PC = PC + 1

- ID: address ← Reg(Rs) + Sign_Extender[Instruction[5:0]], data2 ← Reg(Rt)

- MEM (LW): MemData ← MEM[address]

- MEM (SW): MEM[address] ← data2

- WB (LW Only): Reg(Rt) ← MemData

### I-Type Instructions (Control Flow)

- BEQ: If Reg(Rs) == Reg(Rt), PC = PC + Sign_Extender[Imm], Opcode: 0110

- BNE: If Reg(Rs) != Reg(Rt), PC = PC + Sign_Extender[Imm], Opcode: 0111

- IF: Instruction ← MEM[PC], PC = PC + 1

- ID: data1 ← Reg(Rs), data2 ← Reg(Rt)

- EX (BEQ): EQ ← (data1 == data2)

- EX (BNE): NEQ ← (data1 != data2)

- PC Update (BEQ): If EQ, PC = PC + Imm

- PC Update (BNE): If NEQ, PC = PC + Imm

### J-Type Instructions

- JMP: PC = PC[15:9] | Offset, Opcode: 0001

- CALL: RR = PC + 1; PC = PC[15:9] | Offset, Opcode: 0001

- RET: PC = RR, Opcode: 0001

- IF: Instruction ← MEM[PC]

- ID (JMP, CALL): Offset ← Instruction[8:0]

- PC Update (JMP): PC = Jump_Target

- PC Update (CALL): RR = PC + 1, PC = Jump_Target

- PC Update (RET): PC = RR

### For Instruction

FOR: Initialize loop with Rs as target and Rt as counter, exit when Rt == 0,

Opcode: 1000

- IF: Instruction ← MEM[PC], PC = PC + 1

- ID: Rs ← Loop_Address, Rt ← Counter

- EX: Decrement Rt, Update PC if Rt != 0

## 2.2 The Datapath

Figure 1: DataPath Implementation

The datapath is the core of the processor, responsible for executing instructions by directing the flow of data between various components. It performs arithmetic and logical operations, accesses memory, and updates control signals to ensure proper execution of instructions.

### Overview of the Datapath

The datapath integrates various components, including the Program Counter (PC), Instruction Memory, Register File, ALU, Data Memory, and Control Unit. It supports different instruction types such as R-type (arithmetic/logic), I-type (immediate operations, load/store), and J-type (jump and call).

The datapath executes instructions in five main stages:

- Instruction Fetch (IF): Fetch the next instruction from memory.

- Instruction Decode (ID): Decode the instruction and retrieve necessary operands.

- Execute (EX): Perform arithmetic or logical operations or calculate memory addresses.

- Memory Access (MEM): Access data memory for load/store instructions.

- Write Back (WB): Write results back to the Register File.

## 2.2 Datapath & Control Path

### 2.2.1 Program Counter (PC)

The Program Counter (PC) is responsible for holding the address of the next instruction to execute. It plays a critical role in maintaining the sequential flow of instructions and handling control flow changes.

- Instruction Sequencing: By default, the PC increments by 1 (PC + 1) to fetch the next instruction.

- Control Flow: The PCSource multiplexer determines the next value for the PC:

  - Sequential address (PC + 1).

  - Branch target address for conditional instructions.

  - Jump address for J-Type instructions.

  - Return address from the Return Register (RR).

- Clock and Control:

  - Updates on the rising edge of the clock when PCWrite is asserted.

  - Supports reset to initialize the processor.

The PC ensures accurate instruction fetching and integrates seamlessly with the Instruction Memory, multiplexer, and Control Unit (CU) for efficient execution.

### 2.2.2 Instruction Memory

Instruction Memory is an integral part of the datapath, storing and releasing instructions to be executed by the processor. In the given design, it acts as a read-only memory and instruction fetches are done depending on the value of the Program Counter (PC) at that particular time. In every cycle, an instruction is fetched from the pipeline and moved into the Instruction Register for decoding and execution.

- The Instruction Memory is accessed using the current value of the PC.

- It outputs the instruction corresponding to the PC value, and presents the next sequential PC value (PC+1).

- The fetched instruction is broken down into its component parts: opcode, function code, and operand fields, so that the Control Unit and datapath can correctly decode and execute it.

This component ensures that the processor always gets the right instruction to be executed, acting as the first step in the instruction execution process. With Instruction Memory integrated within the rest of the datapath, the processor will be capable of handling the sequential execution as well as jumps or branches.

### 2.2.3 Instruction Register (IR)

The Instruction Register (IR) holds the instruction fetched from memory during the fetch stage.

- Function: Stores the current instruction for decoding and execution.

- Field Extraction: Breaks the instruction into opcode, function code, and operands.

- Control: Updates on the rising clock edge when IRWrite is asserted.

The IR ensures proper instruction decoding and facilitates communication between the fetch and decode stages.

### 2.2.4 Register File

Register File stores and provides quick access to the processor’s general-purpose registers. In this design, it supports two simultaneous reads and one write in a single clock cycle:

- Read Ports

- Two registers can be read at once by specifying their addresses (Rs1, Rs2).

- The outputs (Reg_out_1 and Reg_out_2) are sent to downstream components such as the ALU or loop control circuitry.

- Write the port.

- If RegWrite is asserted, a single register (Rd) can be written every clock cycle.

- The data to be written (LOAD_DATA) is usually taken from the ALU output or memory (via the memDataReg).

- Decrement Function

- The signal dec_reg_F allows an automatic decrement of one of the registers, supporting loop instructions by reducing the number of iterations tracked.

- Clock and Control

- All read and write operations are synchronized on the rising edge of clk.

- Control signals (RegWrite, dec_reg_F) ensure that data is correctly written or decremented only when instructed by the Control Unit (CU).

This module ensures the processor can retrieve and update register values as needed, forming the heart of the data path’s operand supply and result storage.

### 2.2.5 Arithmetic Logic Unit (ALU)

The Arithmetic Logic Unit (ALU) is the part of the CPU that handles all basic math and logic tasks. It takes two data inputs—usually values from registers—and follows the instruction’s operation code to figure out what to do with them (add, subtract, AND, OR, etc.). After finishing a calculation, the ALU places its output on dedicated lines and updates flags to show whether the result is zero, if a carry was generated, or if the value is negative. By centralizing arithmetic and logic in one unit, instructions execute swiftly, and those status flags help guide subsequent processor operations.

### 2.2.6 Extenders (Sign and Unsigned)

The extenders are responsible for adjusting immediate values to the full 16-bit word size, ensuring compatibility with the datapath.

- Sign Extender: Expands a 6-bit signed immediate value to 16 bits by replicating the sign bit.

- Unsigned Extender: Expands a 6-bit unsigned immediate value to 16 bits by appending zeros to the higher bits.

- Control: The AND_I signal determines whether the sign or unsigned extender is used.

These components ensure immediate values are properly formatted for ALU operations and memory addressing.

### 2.2.7 Data Memory

Data Memory manages all memory-related activities, including loads and stores. The module employs an address (DM_address), which is commonly generated by adding a base register value to a sign-extended offset, to access the required memory location. If a load operation (MemRead) is given, the data from the targeted location is written to the output and recorded by the memDataReg for later transmission to the register file. If a store operation (MemWrite) is indicated, the data from reg_out_2 is written into memory on the next rising clock edge. This setup ensures that both reads and writes occur in a controlled, synchronous manner, maintaining consistency between the memory subsystem and the rest of the processor’s data path.

### 2.2.8 Multiplexers (MUX)

A multiplexer, or MUX, is like a traffic controller for data. It takes multiple inputs and selects one to send forward, based on a control signal. This functionality is critical in the datapath for ensuring data flows to the right places at the right time.

In this project, the multiplexer is used in a variety of ways:

- Program Counter (PC) Updates: The 4-to-1 multiplexer helps decide the next value for the PC. It can select:

- The next sequential address (PC + 1).

- A branch address for conditional instructions.

- A jump address for direct jumps.

- A return address stored in the RR register for function calls.

- Register Selection: Multiplexers decide which registers are used as inputs or outputs during operations. For example:

- The Rs1 value might come from either Rsr or Rsi, depending on the instruction.

- Similar logic applies for Rs2 and Rd selection.

- ALU Inputs: Another multiplexer selects the second input for the ALU. This can be either a value from a register or an extended immediate value, depending on the operation.

- Write-Back Data: The multiplexer here chooses whether the data written back to the register file comes from the ALU result or from memory.

Each multiplexer is controlled by signals generated from the Control Unit (CU). For example:

- When PCSource is set to 00, the PC increments for the next instruction.

- For I-Type instructions, the multiplexer ensures the immediate value is routed to the ALU.

### 2.2.9 Control Unit (CU)

The Control Unit (CU) is responsible for generating control signals and managing the processor's execution flow by interpreting instructions.

- Instruction Decoding:

  - Decodes the opcode and function fields to identify the instruction type (R-Type, I-Type, J-Type).

  - Determines the operation and sets control signals accordingly.

- State Machine:

  - Operates as a finite state machine (FSM) with states for fetch, decode, execute, memory, and write-back stages.

  - Transition between states is driven by the clock signal.

- Generated Signals:

  - PCWrite: Updates the Program Counter (PC).

  - IRWrite: Enables writing to the Instruction Register (IR).

  - MemRead and MemWrite: Control memory operations.

  - RegWrite: Enables register file writes.

  - PCSource: Selects the next PC value (sequential, branch, jump, or return).

  - Flags: Manage ALU operations, load/store instructions, and control instructions (e.g., BEQ, BNE, FOR).

- Behavior for Specific Instructions:

  - For arithmetic operations, signals direct the ALU to perform calculations.

  - For control flow, the CU ensures the correct target address is loaded into the PC.

  - For memory operations, appropriate signals for reading or writing memory are generated.

The CU ensures seamless coordination between the datapath components, allowing the processor to execute instructions efficiently.

### 2.2.10 Performance Registers

The Performance Registers track and record the execution metrics of the processor, providing valuable insights into its operation.

- Metrics Tracked:

  - Total Executed Instructions: Counts all executed instructions.

  - Total Load Instructions: Tracks the number of load (LW) instructions.

  - Total Store Instructions: Tracks the number of store (SW) instructions.

  - Total ALU Instructions: Counts arithmetic and logical operations performed by the ALU.

  - Total Control Instructions: Tracks control flow instructions such as branches (BEQ, BNE) and jumps.

  - Total Clock Cycles: Records the total number of clock cycles elapsed during program execution.

- Control and Update:

  - Metrics are incremented based on flags (flag_executed, flag_load, flag_store, flag_alu, flag_control) generated by the Control Unit.

  - Updates occur synchronously on the rising edge of the clock.

- Integration:

  - Provides real-time monitoring of the processor’s behavior.

  - Metrics can be used for performance analysis and debugging.

The Performance Registers ensure the processor's efficiency and activity are continuously monitored, aiding optimization and verification efforts.

# 3. Control Signals

The Control Unit (CU) manages the operation of the datapath by generating control signals based on the instruction being executed. Each control signal ensures the correct flow of data and the proper execution of each instruction. Below is a detailed description of each control signal and its role.

## 3.1 Control Signals Description

| Signal | Description | Values |
| --- | --- | --- |
| PCWrite | Controls whether the Program Counter (PC) is updated. | 0: Hold current PC. 1: Update PC. |
| MemRead | Enables reading from Data Memory. | 0: No memory read. 1: Read memory. |
| MemWrite | Enables writing to Data Memory. | 0: No memory write. 1: Write to memory. |
| MemToReg | Selects the source of data to write to the Register File. | 1: ALU result. 0: Data Memory. |
| IRWrite | Controls whether the Instruction Register (IR) is updated with the fetched instruction. | 0: No update. 1: Update IR. |
| AND_I | Indicates if the instruction is ANDI to select unsigned immediate values for bitwise operations. | 0: Normal operation. 1: ANDI operation. |
| LOOP | Signals a loop instruction, enabling loop-counter logic. | 0: Not a loop. 1: Loop instruction. |
| enable_RR | Controls whether the Return Register (RR) is updated. | 0: No update. 1: Update RR. |
| flag_executed | Indicates the completion of an instruction execution. | 0: Default state. 1: Execution complete. |
| flag_load | Indicates a load instruction (LW). | 0: Not a load. 1: Load instruction. |
| flag_store | Indicates a store instruction (SW). | 0: Not a store. 1: Store instruction. |
| flag_alu | Indicates an ALU-based instruction. | 0: No ALU operation. 1: ALU operation. |
| flag_control | Indicates a control-flow instruction (e.g., BEQ, BNE, JUMP, FOR). | 0: Not control-flow. 1: Control-flow instruction. |
| PCSource | Determines the source of the next PC value. | 00: Sequential (PC + 1). 01: Branch address. 10: Jump address. 11: Return address. |
| I_type | Indicates whether the instruction is an I-type. | 0: R-type or J-type. 1: I-type. |
| RegWrite | Controls whether the Register File is updated. | 0: No write. 1: Write to the Register File. |

## 3.2 Control Signals Truth Table

The truth table below shows how control signals are set for each instruction type. The values ensure the correct functioning of the datapath for each operation:

| Instr. | PCW | MR | MW | M2R | IRW | A_I | LP | eRR | f_ex | f_ld | f_st | f_alu | f_ctl | PCSc | I_t | RW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LW | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 00 | 1 | 1 |
| SW | 1 | 0 | 1 | X | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 00 | 1 | 0 |
| ADD | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 00 | 0 | 1 |
| SUB | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 00 | 0 | 1 |
| AND | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 00 | 0 | 1 |
| SLL | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 00 | 0 | 1 |
| SLR | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 00 | 0 | 1 |
| ANDI | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 00 | 1 | 1 |
| ADDI | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 00 | 1 | 1 |
| BEQ | 1* | 0 | 0 | X | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 01* | 1 | 0 |
| BNE | 1* | 0 | 0 | X | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 01* | 1 | 0 |
| FOR | 1 | 0 | 0 | X | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 00/01 | 1 | 0 |
| RET | 1 | 0 | 0 | X | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 11 | 0 | 0 |
| J_offset | 1 | 0 | 0 | X | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 10 | 0 | 0 |
| C_offset | 1 | 0 | 0 | X | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 10 | 0 | 0 |